# Tomasulo Engine – Block Specification Sheet

Rev 1.0 · Scope: RV32I integer ALU subset, out-of-order issue/execute, 3 reservation-station entries, 1 common data bus (CDB).

This sheet reconciles every block in the diagram into one consistent interface. Where the diagram is silent or ambiguous, a [DECISION] tag marks the choice made so that all blocks fit together.

## 1. Global Conventions

| Item | Rule |
| --- | --- |
| Clock / reset | Single clock clk, rising-edge. reset is synchronous, active-high. |
| Register file | 32 × 32-bit, x0 hard-wired to 0. Register index = 5 bits. |
| Data width | 32 bits ([31:0]). |
| RSTag | 4-bit enum. NONE=0, ALU1=1, ALU2=2, ALU3=3, MUL1=4, MUL2=5, LOAD1=6, LOAD2=7, STORE1=8, STORE2=9. Only ALU1–3 are used in this version; the rest are reserved. NONE means "value is ready / no producer". |
| Tag ↔ entry | Reservation-station entry i (i = 1..3) is permanently bound to ALU<i>. A tag therefore identifies one RS entry and one functional unit. |
| ALU op (alu_op_t, 4 bit) | ADD=0, SUB=1, SLL=2, SLT=3, SLTU=4, XOR=5, SRL=6, SRA=7, OR=8, AND=9, NOP=15. |
| Timing style | Reads are combinational; state updates happen at the clock edge. FUs and CDB are combinational (no pipeline registers) – see §9 for the cycle model. |
| Python-as-Verilog | One file = one module; ports map 1:1 to the tables below; no dynamic structures, fixed-size arrays only. |

[DECISION] Supported instructions (v1): R-type (ADD, SUB, SLL, SLT, SLTU, XOR, SRL, SRA, OR, AND), I-type ALU (ADDI, SLTI, SLTIU, XORI, ORI, ANDI, SLLI, SRLI, SRAI), LUI, AUIPC. Branches, JAL/JALR, loads, stores, FENCE, ECALL/EBREAK are decoded (so the type is known) but flagged supported=0; the core treats them as bubbles (no RS allocation, no register change, PC continues sequentially). This keeps the datapath exactly as drawn (ALU-only RS) and leaves clean hooks for MUL/LOAD/STORE tags.

## 2. rv32i_types.py – Definitions / Package

Purpose: single source of truth imported by every module (dashed arrow in the diagram). Contains no logic except sign_extend.

Contents

- Opcodes: R=0110011, I_ALU=0010011, I_LOAD=0000011, I_JALR=1100111, S=0100011, B=1100011, LUI=0110111, AUIPC=0010111, J=1101111.

- RSTag: enum from §1.

- alu_op_t: enum from §1.

- DecodedInst (packed struct, produced by frontend, consumed by core):

| Field | Width | Meaning |
| --- | --- | --- |
| valid | 1 | [DECISION, added] Instruction is real. 0 after reset or if memory data is unknown. |
| supported | 1 | [DECISION, added] 1 if it belongs to the v1 subset above. |
| pc | 32 | Address of this instruction. |
| opcode | 7 | inst[6:0]. |
| rd | 5 | inst[11:7]. |
| rs1 | 5 | inst[19:15]. |
| rs2 | 5 | inst[24:20]. |
| funct3 | 3 | inst[14:12]. |
| funct7 | 7 | inst[31:25]. |
| imm | 32 | Sign-extended immediate of the matching format. For LUI/AUIPC it is already inst[31:12] << 12. For shift-immediates it holds the shamt in [4:0]. |
| op_name | 4 | alu_op_t value (see decode table in §4). NOP if unsupported. |

- sign_extend(value, from_bits) → 32-bit sign-extended value.

## 3. Instruction Memory (External)

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| addr | in | 32 | Byte address, word aligned (addr[1:0]=0). |
| inst | out | 32 | Instruction at addr. |

Behaviour: asynchronous (combinational) read, zero wait states. Out-of-range reads return 0x00000013 (NOP, addi x0,x0,0). Read-only during run.

## 4. frontend.py – Fetch + Decode

Purpose: owns the PC, fetches one instruction per cycle, decodes it, and freezes when the core says so.

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| clk, reset | in | 1 | Global. |
| mem_inst | in | 32 | From instruction memory. |
| stall | in | 1 | From core stall_frontend. |
| pc | out | 32 | Current PC; also drives memory addr. |
| next_pc | out | 32 | pc + 4. |
| decoded_inst | out | DecodedInst | Combinational decode of mem_inst with pc attached. |

Functionality

- PC register: reset → RESET_PC (parameter, default 0). On each edge: if (!stall) pc <= next_pc; else hold.

- Fetch: addr = pc (combinational). Because the PC and memory output are stable while stalled, decoded_inst is identical every stalled cycle – the core may re-present the same instruction safely.

- Decode (combinational): field slicing as in §2; immediate by format (I, S, B, U, J); valid=1 after reset release (0 while reset asserted).

- op_name mapping

| Opcode | funct3 | funct7[5] | op_name |
| --- | --- | --- | --- |
| R | 000 | 0 / 1 | ADD / SUB |
| R | 001 / 010 / 011 / 100 | – | SLL / SLT / SLTU / XOR |
| R | 101 | 0 / 1 | SRL / SRA |
| R | 110 / 111 | – | OR / AND |
| I_ALU | 000 | – | ADD (never SUB, ignore imm bit 30) |
| I_ALU | 001 / 010 / 011 / 100 | – | SLL / SLT / SLTU / XOR |
| I_ALU | 101 | 0 / 1 | SRL / SRA (SRLI/SRAI) |
| I_ALU | 110 / 111 | – | OR / AND |
| LUI, AUIPC | – | – | ADD |
| anything else | – | – | NOP, supported=0 |

- Never reads any state outside this module; knows nothing about RS occupancy.

## 5. tomasulo_core.py – Top-Level Control (Issue Logic)

Purpose: the issue stage. Renames registers, gathers operands (from RAT/ARF/CDB), allocates a reservation-station entry, updates the RAT, and stalls the frontend when it cannot issue. It holds no architectural state – it is purely combinational glue (all state lives in RAT, ARF, RS).

Inputs

| Port | Width | Source | Description |
| --- | --- | --- | --- |
| clk, reset | 1 | global | Unused by logic (kept for interface uniformity). |
| decoded_inst | DecodedInst | frontend | Instruction to issue. |
| rat_lookup_tags | 2 × RSTag | RAT (tag1, tag2) | Producer tags for rs1, rs2. |
| arf_read_data | 2 × 32 | ARF (read_data1/2) | Committed values for rs1, rs2. |
| rs_read_data | struct | RS | {has_free (1), free_tag (RSTag)}. [DECISION] The diagram's "rs_read_data" is defined as exactly these two RS outputs. |
| cdb_valid, cdb_tag, cdb_data | 1 / 4 / 32 | CDB | Same-cycle broadcast, used for operand bypass. |

Outputs

| Port | Width | Dest | Description |
| --- | --- | --- | --- |
| rat_lookup_regs | 2 × 5 | RAT lookup_reg1/2 and ARF read_addr1/2 | = {rs1, rs2} of the instruction. (Diagram's "rat_lookup_tags (rs1, rs2)" arrow.) |
| issue_en | 1 | RAT issue_en | 1 when an instruction is actually allocated this cycle. |
| issue_dest | 5 | RAT | = rd. |
| issue_tag | RSTag | RAT | = free_tag chosen by RS. |
| rs_alloc_info → alloc_en, alloc_op, alloc_Vj, alloc_Vk, alloc_Qj, alloc_Qk, alloc_dest | 1/4/32/32/4/4/5 | RS | Bundle written into the free entry. rs_write_en / rs_write_data in the diagram are the same bundle: rs_write_en = alloc_en. |
| rat_update | – | RAT | = {issue_dest, issue_tag} gated by issue_en (not a separate wire). |
| stall_frontend | 1 | frontend | See below. |

Functionality (all combinational, evaluated every cycle)

- issuable = decoded_inst.valid && decoded_inst.supported && rd != 0. (An ALU op targeting x0 has no architectural effect, so it is dropped to save RS slots.)

- Operand 1 (j): LUI → Vj=0, Qj=NONE; AUIPC → Vj=pc, Qj=NONE; otherwise register rs1 via resolve().

- Operand 2 (k): R-type → register rs2 via resolve(); I_ALU, LUI, AUIPC → Vk=imm, Qk=NONE.

- resolve(tag, arf_val)

  - tag == NONE → (V=arf_val, Q=NONE).

  - tag != NONE and cdb_valid && cdb_tag == tag → CDB bypass (V=cdb_data, Q=NONE).

  - otherwise → (V=0, Q=tag) (value will be captured later by the RS).

  - Reading x0 always gives (0, NONE).

- Allocation: alloc_en = issuable && has_free; alloc_op = op_name; alloc_dest = rd.

- Rename: issue_en = alloc_en, issue_dest = rd, issue_tag = free_tag.

- Stall: stall_frontend = issuable && !has_free. Non-issuable instructions (unsupported, rd==x0) never stall – they flow through as bubbles.

- RAT lookups use the pre-update RAT, so add x5,x5,x6 correctly reads the old x5 mapping.

## 6. rat.py – Register Alias Table

Purpose: records, for each architectural register, which in-flight RS entry (if any) will produce its newest value.

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| clk, reset | in | 1 | Reset clears all entries to NONE. |
| issue_en | in | 1 | Rename write enable. |
| issue_dest | in | 5 | Register to rename. |
| issue_tag | in | RSTag | New producer. |
| cdb_valid | in | 1 | Broadcast valid. |
| cdb_tag | in | RSTag | Broadcast producer tag. |
| lookup_reg1, lookup_reg2 | in | 5 | Registers to look up. |
| tag1, tag2 | out | RSTag | Combinational lookup results. |
| cdb_match | out | 1 | [DECISION, added] 1 if any entry's tag equals cdb_tag while cdb_valid. Used by top-level wiring to qualify ARF writes (see §7). |

State: tag[0..31] (RSTag each); tag[0] is forced to NONE.

Functionality

- Lookup: tag1 = tag[lookup_reg1] (and tag2), combinational; x0 → NONE.

- Clear on broadcast (edge): every entry with tag == cdb_tag (and cdb_valid) becomes NONE. The RAT has no cdb_dest port, so this is a tag search across all 32 entries; this is safe because a tag appears in at most one entry (each RS entry has one destination and old mappings are overwritten at rename).

- Rename (edge): if issue_en && issue_dest != 0, tag[issue_dest] <= issue_tag.

- Same-cycle priority: if a clear and a rename hit the same register, the rename wins. (A rename can never collide with the broadcasting tag itself because a broadcasting entry is still busy and is never free_tag.)

- cdb_match is computed from the RAT contents before this edge's updates.

## 7. arf.py – Architectural Register File

Purpose: holds committed register values.

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| clk, reset | in | 1 | Reset zeroes all registers. |
| cdb_valid | in | 1 | Write enable (qualified, see below). |
| cdb_dest | in | 5 | Register to write. |
| cdb_data | in | 32 | Value to write. |
| read_addr1, read_addr2 | in | 5 | Read addresses. |
| read_data1, read_data2 | out | 32 | Combinational read data. |

Functionality

- Write at the edge: if (cdb_valid && cdb_dest != 0) reg[cdb_dest] <= cdb_data. x0 always reads 0.

- No internal read-after-write forwarding. The core's CDB bypass (§5) already covers the same-cycle case, because the RAT still shows the tag in that cycle.

- [DECISION] WAW protection: the ARF's cdb_valid input is driven by the top-level as cdb_valid & rat.cdb_match. A result is committed to the ARF only if it is still the newest producer of that register. Without this, an older instruction that completes after a younger one to the same register would overwrite the correct value. The ARF itself stays unchanged.

## 8. reservation_station.py – Reservation Stations

Purpose: holds the 3 in-flight instructions, waits for missing operands, and presents ready entries to the ALUs.

Entry (RSEntry)

| Field | Width | Meaning |
| --- | --- | --- |
| busy | 1 | Entry in use. |
| op | 4 | alu_op_t. |
| Vj, Vk | 32 | Operand values (valid when matching Q is NONE). |
| Qj, Qk | RSTag | Producer tag, NONE when value present. |
| dest | 5 | Destination architectural register. |

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| clk, reset | in | 1 | Reset clears all busy. |
| alloc_en | in | 1 | Write a new entry. |
| alloc_op, alloc_Vj, alloc_Vk, alloc_Qj, alloc_Qk, alloc_dest | in | 4/32/32/4/4/5 | Entry contents. |
| cdb_valid, cdb_tag, cdb_data | in | 1/4/32 | Broadcast for wake-up and entry release. |
| rs_entries | out | 3 × RSEntry | Entry i → functional_unit ALUi (one-to-one). |
| has_free | out | 1 | [DECISION, added] At least one non-busy entry. Part of core's rs_read_data. |
| free_tag | out | RSTag | [DECISION, added] Lowest-numbered non-busy entry (ALU1 first). |

Functionality (edge-triggered unless noted)

- Free-entry select (comb): free_tag = lowest index with busy==0. An entry that is being released by this cycle's broadcast still counts as busy (conservative, avoids same-entry alloc/release collision).

- Allocate: if alloc_en, write alloc_* into entry free_tag, set busy=1. Core guarantees alloc_en only when has_free.

- Wake-up (capture): for every busy entry: if cdb_valid && Qj == cdb_tag then Vj <= cdb_data; Qj <= NONE; same for Qk/Vk.

- Release: if cdb_valid && cdb_tag == tag_of_entry_i and entry i is busy, busy <= 0. This broadcast is the only way an entry is freed.

- Ready (comb, derived): ready_i = busy_i && Qj_i==NONE && Qk_i==NONE. The entry stays unchanged while waiting for CDB access, so its result remains valid until broadcast.

- No result data is stored; results are recomputed from the entry by the FU each cycle until broadcast.

## 9. functional_unit.py – ALU1 / ALU2 / ALU3 (identical, parameter FU_TAG)

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| clk, reset | in | 1 | Unused by the combinational ALU; kept for a future pipelined FU. |
| rs_entry | in | RSEntry | Entry i from RS. |
| out_valid | out | 1 | busy && Qj==NONE && Qk==NONE. |
| out_tag | out | RSTag | = FU_TAG (ALU1=1, ALU2=2, ALU3=3). |
| out_data | out | 32 | ALU result. |
| out_dest | out | 5 | = rs_entry.dest. |

ALU function (a=Vj, b=Vk, sh=Vk[4:0]):

ADD a+b · SUB a−b · SLL a<<sh · SLT signed a<b → 0/1 · SLTU unsigned a<b → 0/1 · XOR · SRL logical a>>sh · SRA arithmetic a>>>sh · OR · AND · NOP → 0 (out_valid forced 0).

Behaviour: fully combinational, 0 extra cycles after operands are ready. It never changes the RS entry; presenting a result does not consume it. A result is consumed only when the CDB selects it and the RS releases the entry. [DECISION] This removes the need for a CDB grant wire back to the FUs (the diagram has none).

## 10. cdb.py – Common Data Bus

| Port | Dir | Width | Description |
| --- | --- | --- | --- |
| clk, reset | in | 1 | Unused (combinational arbiter). |
| fu1_*, fu2_*, fu3_* | in | valid/tag/data/dest | One group per FU. |
| cdb_valid | out | 1 | OR of the three valids. |
| cdb_tag | out | RSTag | Winner's tag. |
| cdb_data | out | 32 | Winner's data. |
| cdb_dest | out | 5 | Winner's destination. |

Functionality: combinational fixed-priority arbiter, fu1 > fu2 > fu3. Exactly one result per cycle is broadcast; others simply wait (their RS entries stay busy and ready). If no FU is valid, cdb_valid=0 and all consumers ignore tag/data/dest. The bus fans out to ARF (cdb_valid gated by RAT match, cdb_dest, cdb_data), RAT (cdb_valid, cdb_tag), RS (cdb_valid, cdb_tag, cdb_data), and core (cdb_valid, cdb_tag, cdb_data for bypass). Fixed priority cannot starve an entry in v1 because at most one new instruction issues per cycle and a freed entry cannot be reallocated until the cycle after its broadcast; oldest-first arbitration is the recommended upgrade when more FUs are added.

## 11. Top-Level Wiring Summary

The diagram shows tomasulo_core as the control hub; the structural netlist (instances and wires) lives in a thin tomasulo_top.py.

| Signal | From → To |
| --- | --- |
| pc → addr ; inst → mem_inst | frontend ↔ instruction memory |
| decoded_inst ; stall_frontend → stall | frontend ↔ core |
| rat_lookup_regs | core → RAT lookup_reg1/2, ARF read_addr1/2 |
| tag1/tag2 ; read_data1/2 | RAT, ARF → core |
| has_free, free_tag | RS → core |
| alloc_* | core → RS |
| issue_en/dest/tag | core → RAT |
| rs_entries[i] | RS → FU i |
| out_* | FU i → CDB fu<i>_* |
| cdb_* | CDB → RS, RAT, core |
| cdb_valid & rat.cdb_match, cdb_dest, cdb_data | CDB → ARF |

## 12. Cycle Model and Worked Example

Loop-free check: combinational chain is RS regs → FU → CDB → core bypass → alloc → RS D-inputs. No path starts at an RS D-input and returns to an output in the same cycle, so there are no combinational loops. stall_frontend only feeds the PC enable.

Latency: issue in cycle N → entry visible N+1 → execute and broadcast N+1 (if operands ready) → RAT/ARF/RS updated at the end of N+1.

I1: add x3,x1,x2   I2: sub x4,x3,x1   I3: and x5,x2,x2

| Cycle | Events |
| --- | --- |
| 0 | I1 issues: x1, x2 from ARF; RS entry ALU1 allocated; RAT x3 → ALU1. |
| 1 | ALU1 ready, result on CDB (tag ALU1). I2 issues in the same cycle: x3 tag = ALU1 matches CDB, so bypass value and Qj=NONE; allocated to ALU2; RAT x3 cleared (broadcast) and x4 → ALU2 at the edge. ARF[x3] written (RAT matched). |
| 2 | ALU1 freed, ALU2 ready and broadcasts. I3 issues into ALU1 (lowest free). |
| 3 | ALU1 broadcasts I3, ARF[x5] written. |

## 13. Diagram-vs-Spec Reconciliation

| Item | Diagram | Spec decision |
| --- | --- | --- |
| DecodedInst | no valid flag | Added valid, supported. |
| rs_read_data | undefined contents | {has_free, free_tag} from RS. |
| rs_write_en/data vs alloc_* | two names | Same bundle; alloc_* are the RS ports. |
| FU → CDB grant | none drawn | None needed; entries stay until broadcast. |
| RAT clear | only cdb_tag | Clear by tag match; added cdb_match output. |
| ARF write | unconditional | Gated by cdb_match to avoid WAW overwrites. |
| Branch/Load/Store | decoded types only | Treated as bubbles in v1. |

## 14. Verification Checklist

- Reset: RAT all NONE, RS all free, ARF all 0, PC = RESET_PC.

- Independent ops fill ALU1–3, 4th instruction stalls the frontend; PC and decoded_inst hold during stall.

- RAW chain: dependent op waits (Q≠NONE), then wakes on broadcast; CDB-bypass case at issue.

- WAW: add x5,.. (slow, waiting) followed by younger add x5,.. (ready). Final ARF[x5] must be the younger result.

- Three simultaneously ready entries: one broadcast per cycle in priority order, all eventually retire.

- rd = x0 and unsupported instructions: no RS use, no RAT/ARF change, no stall.

- Shifts use only Vk[4:0]; SRAI/SUB selection by funct7[5]; ADDI never decodes to SUB.

- AUIPC uses pc, LUI uses 0 as the first operand.